Arbadell Computer

by Shane Avery

1. Introduction

Arbadell is a computer designed completely from scratch. The purpose of the project is to design a complete computer system to test and develop the skills acquired as a computer engineering graduate at Cal Poly. Also, the project will provide a way to show my skills to an interviewer. Arbadell contains a CPU, RAM, loader, chipset, keyboard input, RS232 interface, and VGA out. The chipset will be design by Elohim Holguin as her senior project.

2. CPU

The CPU will be a CISC like computer. Implementation of the CPU will be done in Verilog. A Xilinx FPGA was chosen for the CPU for a few reasons. First, Xilinx development software is free. I managed to get a hold of the student edition from Professor Sandiage’s book. Secondly, I managed to find a development kit made by Xess that contained a Xilinx chip.

The design of the CPU contains an instruction register to hold the current instruction, an alu (arithmetic and logic unit) that performs add, logical and, logical or, invert, increment, and decrement, a PC (program counter), a stack, a scratch register, a status register, and a control unit.

The CPU is an 8 bit CPU (meaning it has an 8 bit data bus). The address bus is 16 bits. Therefore the CPU can access 65536 bytes of data. The CPU is designed to be run at 4 MHz. Upon reset the CPU will begin executing at address 0x0000. There are 24 instructions. Their names, machine code, and a short description are as follows.

Note: Literal values are 8 bits and memory addresses are 16 bits.

movel		literal value		0x00		move the literal value into the scratch 								register

movem	memory address	0x01		move the byte in the memory address

into the scratch register

moves		memory address	0x02		move the value of the scratch register

into the memory address location

inv					0x03		invert the value of the scratch register

inc					0x04		increments the value of the scratch

register

dec					0x05		decrements the value of the scratch

register

shiftr					0x06		logical shift right of the scratch register

shiftl					0x07		logical shift left of the scratch register

addl		literal value		0x08		add the literal value to the value of the 								scratch register

andl		literal value		0x09		and the literal value to the value of the 								scratch register

orl		literal value		0x0a		or the literal value to the value of the 								scratch register

addm		memory address	0x0b		add the value in the memory address to

the value of the scratch register

andm		memory address	0x0c		and the value in the memory address to

the value of the scratch register

orm		memory address	0x0d		or the value in the memory address to

the value of the scratch register

call		memory address	0x0e		calls the subroutine at memory address

and the current address will be pushed

onto the stack for a return

bra		memory address	0x0f		branches to memory address

braz		memory address	0x10		branches to memory address if the zero 								bit has been set else just continues to 								next instruction

ret					0x11		return from call; will pop of the next 								address on stack and put it in the PC

reti					0x12		return from interrupt

iorecv		literal value		0x13		I/O receive from chipset; the value 								will determine the input received

iosend		literal value    	0x14		I/O send from chipset; the value will 								determine what the chipset will do with 								data given to it

setie		literal value		0x15		this will enable interrupts; keyboard 								interrupt enable will be the value of the 								first bit; serial interrupt enable will be the 							value of the second bit; the interrupt will 								be enabled if the bit is high and cleared  								if low; all other values will be ignored

ti0		memory address	0x16		test interrupt0(keyboard) and branch to 								memory address if set

ti1		memory address 	0x17		test interrupt1(serial in) and branch to 								memory address if set

note: All arithmetic and logical operations store the result into the scratch register.

Interrupts will be handled as follows. The chipset will assert an interrupt pin when it receives data from either the keyboard or serial port. If the interrupts are enabled the CPU will finish its current instruction, save the current memory address and branch to 0xfff0 if a keyboard interrupt and 0xfff8 for a serial interrupt. The CPU will then continue executing instructions and will not interrupt again even if an interrupt pin is asserted by the chipset until the reti instruction has been executed. Alternatively, the user may poll for interrupts using the ti0 and ti1 instructions. However, it is strongly recommended the polling not be used because the chipset will block until interrupts are serviced.

3. Chipset

The chipset was developed by Elohim Holguin as a senior project and was implemented with a PIC microcontroller. It will be involved in most input and outputs to the Arbadell computer. The chipset will be responsible for keyboard in, serial in, serial out, passing data to the video system, and latching the LED array. Whenever the chipset happens to receive a byte from serial or the keyboard it will interrupt the CPU. The interrupt cannot be cleared until the CPU requests to service the interrupt from the chipset.

The communications protocol between the CPU and chipset is a simple two signal protocol. The chipset will look at the IOADDR pin that comes from the CPU. When that pin is asserted the chipset will look at the lower three address lines to determine the action to take. The lower three bits and their action is described as follows.

0x00		keyboard in

0x01		serial in

0x02		serial out

0x03		video out

0x04		latch LED array

0x07		asks the chipset to respond with 0xa5

The last action 0x07 is way for the CPU to determine if the chipset is there and responding. It is meant to be used during the POST (power on self test).

Once IOADDR is asserted the chipset will perform the action and then assert ACK. This will tell the CPU that the operation is complete. If the chipset has put a byte on the data bus the CPU will grab it at this time. The CPU will then clear the IOADDR line telling the chipset that the CPU has received the ACK and, if applicable, has grabbed the byte from the data bus. The chipset in turn will clear ACK and the process will start over. The CPU cannot set the IOADDR line again until the chipset has cleared ACK.

By default the chipset will always have the data bus as input until the IOADDR line is asserted the action is keyboard in, serial in, or respond with 0xa5. Once IOADDR has been cleared the chipset must then switch to input on the data bus before clearing ACK.

4. RAM

Main system memory is a CMOS NVSRAM part. This is so data can be retained even after powerdown.

5. Loader

Somehow the memory has to be initially loaded. An AVR microcontroller will load the RAM with the programs and data before turning the CPU loose on the RAM. The AVR will have an RS232 interface and will connect to the PC via the serial port. The AVR will then be able to accept commands like a dummy terminal. Commands will allow users to load specified block of RAM, read specified blocks of RAM, and pull the CPU out of reset to execute the instructions in RAM.

6. Assembler

The assembler will be written in C. A more detailed document on assembler syntax and usage will be included in the zip file along with the source code.

7. VGA Chip

It would be too complicated to make the chipset directly control the VGA interface. Instead there will be separate VGA chips. The VGA chips will be a CPLD written in verilog and a PIC microcontroller. It will accept commands from the CPU to display characters on a standard VGA monitor.